GPU-Accelerated LLM Inference Engine — Architecture Design (2026)

Technologies: Python, CUDA C++, PyTorch, NCCL

This project is an architecture design for a multi-GPU LLM inference serving system. Rather than a toy demo, the goal was to design a production-grade system from hardware selection through software stack to deployment scripts, with detailed rationale for every decision. This is the kind of design document you would present to a team before starting implementation.

1.1 Problem Statement & Goals

Large Language Models (LLMs) like LLaMA-2 and Mistral are AI models that understand and generate text. They are extremely large — a 7-billion-parameter model needs about 14 GB of GPU memory just to store its weights in FP16. A 13B model needs ~26 GB. The goal of this project is to design a system that can:

Serve LLaMA-2 7B and 13B, and Mistral-7B models for real-time inference (< 100ms first-token latency)

Scale across multiple GPUs when a single GPU cannot hold the full model or meet throughput targets

Achieve at least 1.5x throughput improvement over naive Hugging Face single-GPU inference

Be deployable on both a local workstation (2 GPUs) and a cloud HPC node (4–8 GPUs)

Analogy: imagine you have a very long book to read, but you can only hold 5 pages at a time. If you have 2 friends, you can split the book — each person reads a section and shares the summary. That is essentially what "tensor parallelism" does with GPU memory.

1.2 Key Concepts Explained Simply

What is a GPU?

A GPU (Graphics Processing Unit) is a chip originally designed for rendering graphics, but it turns out to be extremely good at doing many math operations at once. While a CPU might have 8–16 cores, a GPU has thousands of small cores. Deep learning models are mostly matrix multiplication, which GPUs handle very well.

What is CUDA?

CUDA is NVIDIA's programming toolkit that lets you write code that runs on the GPU. You write a "kernel" (a function), and the GPU runs thousands of copies of it in parallel. Example: to add two arrays of 1 million numbers, a CPU does it one by one; a GPU can do thousands simultaneously.

What is a Tensor?

A tensor is just a multi-dimensional array of numbers. A 1D tensor is a list [1, 2, 3]. A 2D tensor is a matrix. Neural networks are built from operations on tensors — that is why the library is called "PyTorch" (Python + Torch/tensors).

What is Tensor Parallelism?

When a model is too big for one GPU, you split each layer's weight matrices across multiple GPUs. Each GPU computes part of the result, then they combine via communication. This is different from "data parallelism" (same model on each GPU, different data).

Same model copied to each GPU, each GPU processes different training examplesData Parallelism:

Model weights are split across GPUs, each GPU holds a piece of the same layerTensor Parallelism:

Different layers of the model live on different GPUs, data flows through like a pipelinePipeline Parallelism:

What is NCCL?

NCCL (pronounced "Nickel") stands for NVIDIA Collective Communication Library. When GPUs need to share data (e.g., combine partial results), NCCL provides optimized functions like AllReduce (sum results from all GPUs and give everyone the answer), Broadcast (send data from one GPU to all), and AllGather (collect pieces from each GPU into a full result on all GPUs). It automatically picks the fastest path — NVLink (direct GPU-to-GPU connection, very fast) or PCIe (the standard slot on the motherboard, slower).

What is Attention / KV-Cache?

Attention is the core mechanism in transformer models (like LLaMA). For each word, the model looks at all previous words to decide what to focus on. This involves computing Keys (K), Values (V), and Queries (Q) — all matrices. The KV-cache stores previously computed K and V so the model does not recompute them for every new token. Without caching, generating 100 tokens would require recomputing attention over all previous tokens each time — very wasteful.

What is NVLink vs. PCIe?

These are two ways GPUs connect to each other inside a machine. PCIe (Peripheral Component Interconnect Express) is the standard slot on a motherboard — PCIe 4.0 gives ~32 GB/s per direction. NVLink is NVIDIA's proprietary high-speed bridge that directly connects two GPUs, giving ~600 GB/s on the A100 (3rd generation NVLink). When GPUs need to exchange data frequently (like in tensor parallelism), NVLink makes a massive difference.

What is Nsight Systems?

NVIDIA Nsight Systems is a profiling tool that records everything happening on the CPU and GPU over time: kernel launches, memory copies, NCCL calls, etc. It produces a timeline visualization so you can see exactly where time is spent and identify bottlenecks. You run it with: nsys profile -o report python your_script.py

1.3 Hardware Architecture

1.3.1 Target Hardware Configurations

This design supports two hardware tiers:

Configuration A — Local Workstation (Development & Testing):

AMD EPYC 7543 (32 cores) or Intel Xeon w5-3435XCPU:

256 GB DDR4 ECC (needed for loading model weights before GPU transfer)RAM:

2x NVIDIA RTX 4090 (24 GB VRAM each, 48 GB total) — connected via PCIe 4.0 x16GPUs:

2 TB NVMe SSD (Samsung 990 Pro) — model weights are 14–26 GB, fast loading mattersStorage:

PCIe 4.0 (~32 GB/s bidirectional). No NVLink on consumer cards.GPU Interconnect:

1200W PSU (each 4090 draws up to 450W)Power:

Configuration B — Cloud/HPC Node (Production):

AMD EPYC 7763 (64 cores) or Intel Xeon Platinum 8380CPU:

512 GB–1 TB DDR4 ECCRAM:

4x NVIDIA A100 80GB SXM4 — connected via NVLink 3.0 (600 GB/s per GPU pair)GPUs:

4 TB NVMe (local scratch) + NFS/Lustre shared filesystem for model weightsStorage:

NVLink 3.0 with NVSwitch for all-to-all GPU connectivityGPU Interconnect:

100 Gbps InfiniBand (if multi-node, not needed for single-node design)Network:

1.3.2 Why These Choices Matter

Memory Budget Analysis for LLaMA-2 7B (FP16):
=================================================
Model weights:              7B params x 2 bytes = 14.0 GB
KV-cache (seq_len=2048):    32 layers x 2(K,V) x 2048 x 4096 x 2 bytes
                            = ~1.1 GB per batch element
Activation memory:          ~0.5 GB (intermediate tensors during forward pass)
PyTorch overhead:           ~1.0 GB (CUDA context, allocator fragmentation)
-------------------------------------------------
Total for batch_size=1:     ~16.6 GB  --> fits on 1x RTX 4090 (24 GB)
Total for batch_size=8:     ~23.8 GB  --> tight on 1x RTX 4090
Total for batch_size=16:    ~31.6 GB  --> needs 2x GPUs with tensor parallelism

Memory Budget Analysis for LLaMA-2 13B (FP16):
=================================================
Model weights:              13B params x 2 bytes = 26.0 GB
KV-cache (seq_len=2048):    40 layers x 2 x 2048 x 5120 x 2 bytes
                            = ~1.6 GB per batch element
-------------------------------------------------
Total for batch_size=1:     ~29.1 GB  --> DOES NOT fit on 1x RTX 4090
                                     --> needs 2x GPUs minimum
                                     --> fits on 1x A100 80GB easily

1.3.3 GPU Interconnect Topology Diagram

Configuration A: PCIe Topology (Local Workstation)
==================================================
         [CPU + System RAM 256GB]
                  |
           [PCIe 4.0 Root Complex]
            /                \
     [PCIe x16]          [PCIe x16]
        |                    |
   [RTX 4090 #0]       [RTX 4090 #1]
     24GB VRAM            24GB VRAM

   GPU-to-GPU bandwidth: ~32 GB/s (via PCIe, through CPU)
   Implication: NCCL AllReduce adds ~0.4ms per MB transferred


Configuration B: NVLink Topology (HPC Node)
==================================================
   [A100 #0] <=== NVLink 3.0 ===> [A100 #1]
      |    \                     /    |
      |     \=== NVLink 3.0 ===/     |
      |              |               |
   NVLink         NVSwitch         NVLink
      |              |               |
      |     /=== NVLink 3.0 ===\     |
      |    /                     \    |
   [A100 #2] <=== NVLink 3.0 ===> [A100 #3]

   GPU-to-GPU bandwidth: ~600 GB/s (direct NVLink)
   Implication: NCCL AllReduce adds ~0.02ms per MB transferred
   That is 20x faster than PCIe!

1.4 Software Architecture

1.4.1 Software Stack (Bottom to Top)

Layer 5: [User Application / REST API]     <-- FastAPI or gRPC endpoint
           |
Layer 4: [Inference Orchestrator]           <-- Python: request batching,
           |                                    scheduling, tokenization
Layer 3: [Model Runtime]                    <-- PyTorch + custom modules,
           |                                    tensor parallel wrapper
Layer 2: [Communication Layer]              <-- NCCL 2.18+ (AllReduce,
           |                                    AllGather, Broadcast)
Layer 1: [GPU Compute Layer]                <-- CUDA 12.x kernels,
           |                                    cuBLAS (matrix multiply),
           |                                    custom fused kernels
Layer 0: [Hardware]                         <-- NVIDIA GPUs, NVLink/PCIe,
                                                CPU, RAM, NVMe storage

1.4.2 Detailed Software Versions & Dependencies

requirements.txt / environment:
================================
# OS
Ubuntu 22.04 LTS (or RHEL 8.x for HPC)

# NVIDIA Driver & CUDA
NVIDIA Driver:     >= 535.104
CUDA Toolkit:      12.1.1
cuDNN:             8.9.7
NCCL:              2.18.3

# Python Stack
Python:            3.10.x
PyTorch:           2.2.0+cu121
transformers:      4.38.x  (Hugging Face)
accelerate:        0.27.x  (Hugging Face multi-GPU)
safetensors:       0.4.x   (fast weight loading)
tokenizers:        0.15.x  (Rust-based fast tokenizer)

# Custom CUDA
NVCC:              12.1 (for compiling .cu files)
Nsight Systems:    2023.4+ (profiling)
Nsight Compute:    2023.3+ (kernel-level profiling)

# Serving
FastAPI:           0.109.x
uvicorn:           0.27.x
pydantic:          2.6.x

1.4.3 Component Design

Component 1: Model Loader

Responsible for loading model weights from disk and distributing them across GPUs according to the tensor parallelism strategy.

# model_loader.py - Loads and shards model weights across GPUs
import torch
from safetensors.torch import load_file
from pathlib import Path

class TensorParallelModelLoader:
    """
    Loads a Hugging Face model and splits weight matrices
    across multiple GPUs for tensor parallelism.

    How tensor parallelism splitting works:
    - For each linear layer (e.g., q_proj, k_proj, v_proj, o_proj),
      the weight matrix has shape [output_dim, input_dim].
    - We split along the OUTPUT dimension (rows), so each GPU
      holds (output_dim / N) rows, where N = number of GPUs.
    - Each GPU computes a partial result, then we use NCCL
      AllGather to combine them.
    """

    def __init__(self, model_path: str, world_size: int, rank: int):
        self.model_path = Path(model_path)
        self.world_size = world_size  # total number of GPUs
        self.rank = rank              # this GPU's index (0, 1, ...)
        self.device = torch.device(f"cuda:{rank}")

    def load_sharded_weights(self):
        """Load only this GPU's shard of each weight tensor."""
        state_dict = load_file(self.model_path / "model.safetensors")
        sharded = {}

        for name, tensor in state_dict.items():
            if self._should_shard(name):
                # Split along output dimension
                chunk_size = tensor.shape[0] // self.world_size
                start = self.rank * chunk_size
                end = start + chunk_size
                sharded[name] = tensor[start:end, :].to(self.device)
            else:
                # Replicate non-shardable layers (e.g., LayerNorm)
                sharded[name] = tensor.to(self.device)

        return sharded

    def _should_shard(self, name: str) -> bool:
        """Only shard large linear layers, not LayerNorm or embeddings."""
        shard_keywords = ["q_proj", "k_proj", "v_proj", "o_proj",
                          "gate_proj", "up_proj", "down_proj"]
        return any(kw in name for kw in shard_keywords)

Component 2: Tensor Parallel Attention Layer

Wraps the standard attention computation to work across multiple GPUs using NCCL.

# tensor_parallel_attention.py
import torch
import torch.nn as nn
import torch.distributed as dist

class TensorParallelAttention(nn.Module):
    """
    Attention layer split across multiple GPUs.

    Architecture:
    1. Each GPU holds 1/N of Q, K, V projection weights
    2. Each GPU computes attention on its local heads
    3. After attention, NCCL AllReduce combines the output
       projection results across all GPUs

    For LLaMA-2 7B:
      - 32 attention heads, hidden_dim = 4096
      - head_dim = 4096 / 32 = 128
      - With 2 GPUs: each GPU handles 16 heads
      - With 4 GPUs: each GPU handles 8 heads
    """

    def __init__(self, hidden_dim, num_heads, world_size, rank):
        super().__init__()
        self.num_heads = num_heads
        self.local_heads = num_heads // world_size  # heads per GPU
        self.head_dim = hidden_dim // num_heads
        self.local_dim = self.local_heads * self.head_dim

        # Each GPU only has weights for its local heads
        self.q_proj = nn.Linear(hidden_dim, self.local_dim, bias=False)
        self.k_proj = nn.Linear(hidden_dim, self.local_dim, bias=False)
        self.v_proj = nn.Linear(hidden_dim, self.local_dim, bias=False)
        self.o_proj = nn.Linear(self.local_dim, hidden_dim, bias=False)

    def forward(self, x, kv_cache=None):
        # x shape: [batch, seq_len, hidden_dim]
        q = self.q_proj(x)  # [batch, seq_len, local_dim]
        k = self.k_proj(x)
        v = self.v_proj(x)

        # Reshape for multi-head attention
        # [batch, seq_len, local_heads, head_dim]
        q = q.view(*q.shape[:2], self.local_heads, self.head_dim)
        k = k.view(*k.shape[:2], self.local_heads, self.head_dim)
        v = v.view(*v.shape[:2], self.local_heads, self.head_dim)

        # Update KV-cache (append new K, V to cache)
        if kv_cache is not None:
            k = torch.cat([kv_cache[0], k], dim=1)
            v = torch.cat([kv_cache[1], v], dim=1)
        new_cache = (k, v)

        # Scaled dot-product attention (PyTorch 2.0 built-in)
        # This uses FlashAttention under the hood
        attn_out = torch.nn.functional.scaled_dot_product_attention(
            q.transpose(1, 2), k.transpose(1, 2), v.transpose(1, 2),
            is_causal=True
        ).transpose(1, 2).contiguous()

        # Flatten heads back: [batch, seq_len, local_dim]
        attn_out = attn_out.view(*x.shape[:2], self.local_dim)

        # Output projection (partial result on this GPU)
        output = self.o_proj(attn_out)

        # NCCL AllReduce: sum partial outputs from all GPUs
        # After this, every GPU has the same full output
        dist.all_reduce(output, op=dist.ReduceOp.SUM)

        return output, new_cache

Component 3: KV-Cache Manager

Manages the Key-Value cache across the generation process. The cache grows with each new token and must be tracked per GPU.

# kv_cache.py
import torch
from dataclasses import dataclass

@dataclass
class KVCacheConfig:
    """Configuration for KV-cache allocation."""
    num_layers: int       # 32 for LLaMA-2 7B, 40 for 13B
    num_local_heads: int  # heads per GPU
    head_dim: int         # 128 for LLaMA-2
    max_seq_len: int      # maximum sequence length (e.g., 2048)
    dtype: torch.dtype = torch.float16

class KVCacheManager:
    """
    Pre-allocates GPU memory for KV-cache to avoid
    fragmentation during generation.

    Memory per layer = 2(K,V) x max_seq_len x local_heads x head_dim x 2 bytes
    Total for LLaMA-2 7B on 2 GPUs:
      = 32 layers x 2 x 2048 x 16 heads x 128 dim x 2 bytes
      = 32 x 2 x 2048 x 16 x 128 x 2 = 537 MB per GPU
    """

    def __init__(self, config: KVCacheConfig, device: torch.device):
        self.config = config
        self.device = device
        self.cache = self._allocate()
        self.current_len = 0  # tokens generated so far

    def _allocate(self):
        """Pre-allocate empty cache tensors for all layers."""
        cache = []
        for _ in range(self.config.num_layers):
            k = torch.zeros(
                1, self.config.max_seq_len,
                self.config.num_local_heads, self.config.head_dim,
                dtype=self.config.dtype, device=self.device
            )
            v = torch.zeros_like(k)
            cache.append((k, v))
        return cache

    def update(self, layer_idx, new_k, new_v):
        """Append new K, V to the cache for a given layer."""
        pos = self.current_len
        seq = new_k.shape[1]
        self.cache[layer_idx][0][:, pos:pos+seq] = new_k
        self.cache[layer_idx][1][:, pos:pos+seq] = new_v

    def get(self, layer_idx):
        """Return cached K, V up to current position."""
        return (
            self.cache[layer_idx][0][:, :self.current_len],
            self.cache[layer_idx][1][:, :self.current_len]
        )

    def advance(self, num_tokens=1):
        self.current_len += num_tokens

    def reset(self):
        self.current_len = 0

Component 4: Custom CUDA Kernel — Fused Scale + Mask + Softmax

In the standard attention computation, there are 3 separate operations: (1) scale the attention scores, (2) apply causal mask, (3) compute softmax. Each operation launches a separate GPU kernel, with overhead for each launch. A "fused" kernel combines all 3 into a single GPU launch, reducing overhead.

// fused_attention.cu
// Fused Scale + Causal Mask + Softmax CUDA kernel
//
// Why fuse these operations?
// - Each kernel launch has ~5-10 microsecond overhead
// - With seq_len=2048 and 32 layers, that is 3 ops x 32 layers
//   = 96 kernel launches just for attention score processing
// - Fusing into 1 kernel: 32 launches (3x reduction in overhead)
// - Also saves memory bandwidth: intermediate results stay in
//   GPU registers instead of being written to global memory

#include <cuda_runtime.h>
#include <cuda_fp16.h>
#include <math.h>

// Thread block configuration:
//   - blockDim.x = 256 (threads per block)
//   - gridDim.x = ceil(seq_len / 256)
//   - gridDim.y = batch_size * num_heads
//   Each thread processes one row of the attention matrix

__global__ void fused_scale_mask_softmax_kernel(
    float* __restrict__ scores,   // [batch*heads, seq_q, seq_k]
    const int seq_q,              // query sequence length
    const int seq_k,              // key sequence length
    const float scale,            // 1/sqrt(head_dim) = 1/sqrt(128)
    const int current_pos         // for causal mask boundary
) {
    // Each thread handles one query position (one row)
    int row = blockIdx.x * blockDim.x + threadIdx.x;
    int head_batch = blockIdx.y;

    if (row >= seq_q) return;

    float* row_ptr = scores + head_batch * seq_q * seq_k + row * seq_k;

    // Step 1: Scale + Mask + find max (for numerical stability)
    float max_val = -1e9f;
    for (int col = 0; col < seq_k; col++) {
        // Scale
        float val = row_ptr[col] * scale;
        // Causal mask: mask out future tokens
        if (col > current_pos + row) {
            val = -1e9f;  // will become 0 after softmax
        }
        row_ptr[col] = val;
        max_val = fmaxf(max_val, val);
    }

    // Step 2: Compute exp(x - max) and sum
    float sum = 0.0f;
    for (int col = 0; col < seq_k; col++) {
        float val = expf(row_ptr[col] - max_val);
        row_ptr[col] = val;
        sum += val;
    }

    // Step 3: Normalize (divide by sum)
    float inv_sum = 1.0f / (sum + 1e-6f);
    for (int col = 0; col < seq_k; col++) {
        row_ptr[col] *= inv_sum;
    }
}

// Launch configuration:
// dim3 grid(ceil(seq_q / 256), batch_size * num_heads);
// dim3 block(256);
// fused_scale_mask_softmax_kernel<<<grid, block>>>(
//     scores, seq_q, seq_k, 1.0f/sqrtf(128.0f), current_pos);

Component 5: Inference Orchestrator

The main script that ties everything together: loads the model, manages the generation loop, and handles the NCCL process group.

# inference_engine.py - Main orchestrator
import os
import torch
import torch.distributed as dist
from model_loader import TensorParallelModelLoader
from kv_cache import KVCacheManager, KVCacheConfig

class InferenceEngine:
    """
    Orchestrates the full inference pipeline:
    1. Initialize NCCL process group
    2. Load sharded model weights
    3. Pre-allocate KV-cache
    4. Run autoregressive generation loop
    """

    def __init__(self, model_path: str):
        # Step 1: Initialize NCCL
        dist.init_process_group(backend="nccl")
        self.rank = dist.get_rank()
        self.world_size = dist.get_world_size()
        self.device = torch.device(f"cuda:{self.rank}")
        torch.cuda.set_device(self.device)

        # Step 2: Load model
        loader = TensorParallelModelLoader(
            model_path, self.world_size, self.rank
        )
        self.weights = loader.load_sharded_weights()
        self.model = self._build_model()  # construct layers

        # Step 3: Pre-allocate KV-cache
        self.kv_cache = KVCacheManager(
            KVCacheConfig(
                num_layers=32,
                num_local_heads=32 // self.world_size,
                head_dim=128,
                max_seq_len=2048
            ),
            self.device
        )

    @torch.no_grad()
    def generate(self, input_ids, max_new_tokens=100):
        """Autoregressive token generation loop."""
        self.kv_cache.reset()
        tokens = input_ids.to(self.device)

        for step in range(max_new_tokens):
            # Forward pass through all layers
            logits = self.model.forward(
                tokens if step == 0 else tokens[:, -1:],
                self.kv_cache
            )
            self.kv_cache.advance(
                tokens.shape[1] if step == 0 else 1
            )

            # Sample next token (greedy for simplicity)
            next_token = logits[:, -1, :].argmax(dim=-1, keepdim=True)
            tokens = torch.cat([tokens, next_token], dim=1)

            # Check for end-of-sequence
            if next_token.item() == 2:  # EOS token
                break

        return tokens

1.5 Full Architecture Diagram

=================================================================
                FULL SYSTEM ARCHITECTURE
=================================================================

[Client / REST API]  POST /generate {"prompt": "...", "max_tokens": 100}
        |
        v
[FastAPI Server]  (uvicorn, async, runs on CPU)
        |
        v
[Tokenizer]  (HuggingFace tokenizers, Rust-based, CPU)
   "Hello world" --> [15043, 3186]
        |
        v
[Inference Orchestrator]  (Python, coordinates the generation loop)
        |
        |  Distributes to GPU processes via torch.distributed
        |
   +----+----+
   |         |
   v         v
[GPU 0]    [GPU 1]       (each runs a process with rank 0, 1)
   |         |
   |  Model Layers (sharded via tensor parallelism):
   |  +-----------------------------------------+
   |  | Embedding: replicated on both GPUs      |
   |  | Layer 0-31: Q,K,V,O projections sharded |
   |  |   GPU 0 holds heads 0-15                |
   |  |   GPU 1 holds heads 16-31               |
   |  | LayerNorm: replicated on both GPUs      |
   |  | LM Head: replicated on both GPUs        |
   |  +-----------------------------------------+
   |         |
   |  For each transformer layer:
   |    1. Each GPU computes local attention (its heads)
   |    2. NCCL AllReduce on attention output
   |    3. Each GPU computes local FFN (its shard)
   |    4. NCCL AllReduce on FFN output
   |         |
   |  [KV-Cache per GPU]
   |    GPU 0: caches K,V for heads 0-15
   |    GPU 1: caches K,V for heads 16-31
   |    Memory: ~537 MB per GPU (at max seq_len)
   |         |
   +----+----+
        |
        v
[Token Sampler]  (greedy / top-k / top-p sampling)
        |
        v
[Detokenizer]  (token IDs --> text)
        |
        v
[Response]  {"text": "The document discusses...", "tokens": 47}

=================================================================
  NCCL Communication Pattern (per transformer layer):
=================================================================
  GPU 0: partial_attn_out ---+
                              |--> AllReduce(SUM) --> full_attn_out
  GPU 1: partial_attn_out ---+

  GPU 0: partial_ffn_out  ---+
                              |--> AllReduce(SUM) --> full_ffn_out
  GPU 1: partial_ffn_out  ---+

  Total NCCL calls per token: 2 AllReduce x 32 layers = 64 calls
  Data per AllReduce: hidden_dim x batch x 2 bytes
                    = 4096 x 1 x 2 = 8 KB (very small, latency-bound)
=================================================================

1.6 Launch & Deployment Scripts

1.6.1 Launch Script (Local 2-GPU Workstation)

#!/bin/bash
# launch_local.sh - Launch inference on 2 local GPUs
# Usage: bash launch_local.sh

# Verify GPUs are visible
echo "Available GPUs:"
nvidia-smi --query-gpu=index,name,memory.total --format=csv,noheader

# Set NCCL environment variables
export NCCL_DEBUG=WARN              # set to INFO for debugging
export NCCL_P2P_LEVEL=NVL           # prefer NVLink if available
export NCCL_IB_DISABLE=1            # no InfiniBand on workstation
export CUDA_VISIBLE_DEVICES=0,1     # use GPU 0 and 1

# Launch with torchrun (PyTorch distributed launcher)
# torchrun sets RANK, LOCAL_RANK, WORLD_SIZE automatically
torchrun \
    --standalone \
    --nproc_per_node=2 \
    inference_engine.py \
    --model-path ./models/llama-2-7b-hf \
    --max-seq-len 2048 \
    --dtype float16

1.6.2 Launch Script (HPC Node with SLURM, 4 GPUs)

#!/bin/bash
# launch_hpc.slurm - SLURM job script for 4-GPU A100 node
# Usage: sbatch launch_hpc.slurm

#SBATCH --job-name=llm-inference
#SBATCH --nodes=1
#SBATCH --ntasks-per-node=4
#SBATCH --gres=gpu:4
#SBATCH --cpus-per-task=16
#SBATCH --mem=256G
#SBATCH --time=00:30:00
#SBATCH --partition=gpu
#SBATCH --output=inference_%j.log

# Load modules (HPC cluster specific)
module load cuda/12.1
module load nccl/2.18.3
module load python/3.10

# NCCL environment for NVLink
export NCCL_DEBUG=WARN
export NCCL_P2P_LEVEL=NVL
export NCCL_SOCKET_IFNAME=ib0      # InfiniBand interface

# Launch on 4 GPUs
srun torchrun \
    --standalone \
    --nproc_per_node=4 \
    inference_engine.py \
    --model-path /scratch/models/llama-2-13b-hf \
    --max-seq-len 2048 \
    --dtype float16

1.6.3 Benchmarking Script

#!/bin/bash
# benchmark.sh - Run standardized benchmarks and collect metrics
# Measures: first-token latency, per-token latency, throughput

echo "=== LLM Inference Benchmark Suite ==="
echo "Date: $(date)"
echo "GPUs: $(nvidia-smi --query-gpu=name --format=csv,noheader | head -1)"
echo "Num GPUs: $1"
echo ""

# Test configurations
MODELS=("llama-2-7b" "llama-2-13b" "mistral-7b")
BATCH_SIZES=(1 4 8 16)
SEQ_LENS=(128 512 1024 2048)
NUM_GPUS=$1

for model in "${MODELS[@]}"; do
    for bs in "${BATCH_SIZES[@]}"; do
        for seq in "${SEQ_LENS[@]}"; do
            echo "--- $model | batch=$bs | seq_len=$seq | gpus=$NUM_GPUS ---"
            torchrun --nproc_per_node=$NUM_GPUS \
                benchmark_runner.py \
                --model $model \
                --batch-size $bs \
                --seq-len $seq \
                --warmup-steps 5 \
                --benchmark-steps 20 \
                --output results/${model}_bs${bs}_seq${seq}_gpu${NUM_GPUS}.json
        done
    done
done

echo "=== Benchmark Complete ==="
echo "Results saved to ./results/"

1.6.4 Profiling Script

#!/bin/bash
# profile.sh - Profile with NVIDIA Nsight Systems
# Produces a .nsys-rep file you can open in Nsight Systems GUI

export NCCL_DEBUG=INFO  # show NCCL communication details

# Profile a single inference run
nsys profile \
    --trace=cuda,nvtx,osrt \
    --cuda-memory-usage=true \
    --gpu-metrics-device=all \
    --output=profile_report \
    torchrun --nproc_per_node=2 \
        inference_engine.py \
        --model-path ./models/llama-2-7b-hf \
        --benchmark-mode \
        --num-tokens 50

echo "Profile saved to profile_report.nsys-rep"
echo "Open with: nsys-ui profile_report.nsys-rep"

# What to look for in the profile:
# 1. NCCL AllReduce bars: are they overlapping with compute?
# 2. Kernel durations: which kernels take the most time?
# 3. Memory copies: any unnecessary CPU<->GPU transfers?
# 4. GPU idle gaps: are GPUs waiting on communication?

1.7 Performance Design Targets

Expected Performance Comparison:
================================================================
Setup                      | Throughput      | First-Token
                           | (tokens/sec)    | Latency
================================================================
Baseline (HF, 1x 4090)    |  ~35 tok/s      | ~120 ms
Tensor Parallel (2x 4090) |  ~55 tok/s      | ~85 ms
TP + Fused Kernels (2x)   |  ~65 tok/s      | ~70 ms
TP (4x A100, NVLink)      |  ~120 tok/s     | ~40 ms
TP + Fused (4x A100)      |  ~150 tok/s     | ~30 ms
================================================================

Key bottlenecks by configuration:
  PCIe (workstation): NCCL communication latency (~0.4ms per AllReduce)
  NVLink (HPC):       Compute-bound (attention + FFN kernels)

Memory usage (LLaMA-2 7B, FP16, batch=1):
  1 GPU:   ~16.6 GB
  2 GPUs:  ~8.8 GB per GPU (weights split) + 0.5 GB cache
  4 GPUs:  ~4.9 GB per GPU (weights split) + 0.3 GB cache